# VLSI Design Internship – Task 3 Report

Topic: Verilog RTL Design of Sequential Circuits and Flip-Flops

Name: HARSH SAXENA
College: USICT (GGSIPU)

# D Flip-Flop simulation output

CLK is toggling every 5 time units.

D changes at 10, 20, 30, and 40 time units.

Q updates only on the positive (rising) edge of the clock, which is the correct behavior of a D Flip-Flop.

The red section at the beginning of Q is normal—it indicates Q is unknown (X) before the first rising clock edge because it hasn't been initialized yet.

D Flip-Flop Simulation Truth Table (According to Waveform)

| Time (ns) | CLK ↑ | D | Q (Output) |
| --- | --- | --- | --- |
| 0 | No | 0 | X (Unknown) |
| S5 | ↑ | 0 | 0 |
| 15 | ↑ | 1 | 1 |
| 25 | ↑ | 0 | 0 |
| 35 | ↑ | 1 | 1 |

# J-K Flip-Flop simulation output

Signals

- CLK → Clock input

- J → J input

- K → K input

- Q → Output

The JK Flip-Flop changes its output only at the positive (rising) edge of the clock.

Waveform Explanation

Initial State (0–10 ns)

J = 0

K = 0

Q = X (Unknown)

The red portion of Q indicates an unknown value because the flip-flop has not yet received a rising clock edge.

At the first positive clock edge, since J = 0 and K = 0, the flip-flop is in the No Change condition. Because there was no previous valid output, Q remains unknown.

10–20 ns

J = 0

K = 1

This is the Reset condition.

At the next rising edge of the clock:

Q = 0

The output is reset to logic 0.

20–30 ns

J = 1

K = 0

This is the Set condition.

At the next positive clock edge:

Q = 1

The output becomes logic 1.

30–40 ns

J = 1

K = 1

This is the Toggle condition.

At the next rising edge:

Previous Q = 1

New Q = 0

The output toggles from 1 to 0.

JK Flip-Flop Truth Table

| J | K | Operation | Next Q |
| --- | --- | --- | --- |
| 0 | 0 | No Change | Previous Q |
| 0 | 1 | Reset | 0 |
| 1 | 0 | Set | 1 |
| 1 | 1 | Toggle | Complement of Previous Q |

JK Flip-Flop Simulation Truth Table

| Time (ns) | CLK ↑ | J | K | Previous Q | New Q | Operation |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | ↑ | 0 | 0 | X | X | No Change |
| 15 | ↑ | 1 | 1 | X | Toggle (undefined initially) |  |
| 25 | ↑ | 0 | 0 | 1 | 1 | No Change |
| 35 | ↑ | 1 | 1 | 1 | 0 | Toggle |

Q change only at the clock edge:

The JK Flip-Flop is positive edge-triggered, so it samples the values of J and K only when the clock changes from 0 to 1. Between clock edges, even if J or K changes, the output Q remains unchanged.

# 4-bit Register simulation

Signals

- CLK → Clock signal

- D[3:0] → 4-bit input

- Q[3:0] → 4-bit output

Waveform Explanation

Initially:

CLK = 0

D = 0

Q = X

- Q = X (red) means the register is uninitialized because no clock edge has occurred yet.

First Rising Edge (≈5 ns)

D = 0

Clock rises from 0 → 1.

Therefore,

Q = 0

The register stores the value 0000.

Second Rising Edge (≈15 ns)

Before the clock edge,

D = 3

Binary:

3 = 0011

At the positive edge,

Q = 3

The register stores 0011.

Third Rising Edge (≈25 ns)

Input changes to

D = A

Hexadecimal:

A = 1010

At the next rising edge,

Q = A

The register stores 1010.

Fourth Rising Edge (≈35 ns)

Input becomes

D = F

Hexadecimal:

F = 1111

After the rising edge,

Q = F

The register stores 1111.

Observation

The important thing to notice is:

- When D changes, Q does not change immediately.

- Q changes only at the rising edge of CLK.

This is exactly how a register works.

Timing Table

| Clock Edge | D (Input) | Q (Output) |
| --- | --- | --- |
| Initial | 0 | X (Unknown) |
| 1st Rising Edge | 0 | 0 |
| 2nd Rising Edge | 3 | 3 |
| 3rd Rising Edge | A | A |
| 4th Rising Edge | F | F |

Conclusion

The simulation verifies the correct operation of the 4-bit register. The output Q stores the value of the input D only at the positive edge of the clock and retains the stored value until the next rising edge. The initial unknown state (X) is due to the register not being initialized before the first clock edge. The waveform confirms that the register correctly captures the input values 0, 3, A, and F on successive clock pulses.

# 4-bit Binary Counter simulation

Signals Used

| Signal | Description |
| --- | --- |
| CLK | Clock Input |
| Q[3:0] | 4-bit Counter Output |

Initial State

At the beginning,

CLK = 0

Q = 0000

The counter starts from 0.

First Rising Edge

When the first positive edge of the clock occurs,

Q = 0001

The counter increments from 0 to 1.

Second Rising Edge

Q = 0010

Counter becomes 2.

Third Rising Edge

Q = 0011

Counter becomes 3.

Counting Sequence

The counter continues increasing on every positive clock edge.

| Decimal | Binary | GTKWave Display |
| --- | --- | --- |
| 0 | 0000 | 0 |
| 1 | 0001 | 1 |
| 2 | 0010 | 2 |
| 3 | 0011 | 3 |
| 4 | 0100 | 4 |
| 5 | 0101 | 5 |
| 6 | 0110 | 6 |
| 7 | 0111 | 7 |
| 8 | 1000 | 8 |
| 9 | 1001 | 9 |
| 10 | 1010 | A |
| 11 | 1011 | B |
| 12 | 1100 | C |
| 13 | 1101 | D |
| 14 | 1110 | E |
| 15 | 1111 | F |

Notice that GTKWave displays values greater than 9 in hexadecimal:

- A = 10

- B = 11

- C = 12

- D = 13

- E = 14

- F = 15

Counter Overflow

After reaching

1111 (15)

the next positive clock edge causes

1111 → 0000

The counter automatically starts counting from 0 again.

This is because a 4-bit counter can represent only 16 values (0–15).

Observation

- The counter increments by one on every positive edge of the clock.

- Between clock edges, the output remains unchanged.

- The output follows the sequence 0 → 1 → 2 → … → 15.

- After reaching 15, it wraps around to 0 and continues counting.

- The waveform confirms the correct operation of the 4-bit synchronous binary counter.

Conclusion

The simulation successfully verifies the operation of the 4-bit binary counter. The output increments by one at every positive edge of the clock and counts sequentially from 0 to 15. After reaching the maximum value (1111), the counter automatically resets to 0000 and repeats the counting sequence. The GTKWave waveform confirms the expected behavior of a synchronous 4-bit counter.